METEOROLOGICAL SYSTEMS

VECTOR-M Aerological Radar Computation Complex (ARCC)

RZM Meteorological Small-Sized Radiosonde

VECTOR-M (ARCC)

Purpose

VECTOR-M Aerological Radar Computation Complex (ARCC) is designed for temperature-winds air sounding by releasing and tracking radiosondes in the free-flight mode. The article ensures the pre-flight check, automatic tracking of radiosondes in flight, reception and processing of coordinate-telemetry data, formation and transmission of aerological telegrams via communication channels to the data user.

VECTOR-M ARCC is classed as a digital air radio-sounding system with automatic data processing.

Due to its high automation level the system ensures:

→ automatic functional test to automatically check parameters of the antenna system and follower drive;

→ system remote control by one operator (PC-based operator's working station);

→ automatic processing of secondary meteo data and issuing of meteo bulletins via communication channels, as well as storage of aerological data and meteo telegrams in archive for further analysis.

Performance

| Minimum range (to radiosonde release point), m | 20 |
| --- | --- |
| Maximum sounding range, m, min | 250 000 |
| Maximum sounding height, m, min | 40 000 |
| Meteo parameter range: |  |
| air temperature, 0C | from -90 to +50 |
| relative humidity, % | from 0 to 100 |
| actual and average wind velocity, m/s | from 0 to 150 |
| actual wind direction, grad | from 0 to 360 |
| Electric power supply, AC, single phase, V, Hz | 220, 50 |
| Average power consumption, W, max | 500 |
| Overall dimensions of antenna curtain, m, max | 1.8 х 1.8 |
| VECTOR-M ARCC weight: |  |
| antenna post, kg, max | 230 |
| operator's post, kg, max | 40 |

Certificate of Conformity No.: РОСС RU.ME88.H00468, valid from May 05, 2015 until May 05, 2017, issued by the Federal Agency for Technical Regulation and Metrology

RZM – 2

Purpose

Meteorological small-sized radiosonde (series RZM) is intended for operation as part of the complex air sounding systems

The RZM-2-16 and RZM-2-17 versions employ the digital processing of telemetry information

Radiosonde ensures:

- full compatibility with stations AVK-1, AVK -1М, Vector-М, Marl-A;

- 	temperature range of minus 900 С to plus 500 С with a maximum permissible error of 1.00С;

-	relative humidity range of 0 to 100% with a maximum permissible error of 10% of relative humidity;

- continuous operation for 3 hours;

- radio unit mean life of 30 hours, min

Performance:

| Radiosonde carrier frequency, MHz | 1680±8, 1780±8 |
| --- | --- |
| RF Pulse-repetition frequency, kHz | 800±25 |
| RF Pulse-repetition frequency deviation, kHz | 12 - 18 |
| Radiating power flux density, W/m2, min | 1.5 х 10-3 |
| Interrogation radio pulse sensitivity, dB, max | minus 60 |
| The sequence of the channels in the telemetering cycle is as follows: temperature –– reference –– temperature –humidity | Т – R – Т - H |
| The pulse repetition period, s |  |
| in the reference channel | 1449 - 1785 |
| in the temperature channel | 1562 - 58821 |
| in the humidity channel | 2000 - 3500 |
| The shaper output pulses duration, s |  |
| in the reference channel | 200 - 350 |
| in the meteo data channels | 435 х 765 |
| Channel interval duration, s | 5.1 – 5.4 |
| Weight, kg, max | 0.36 |
| Overall dimensions, mm | 267 х 145 х 161 |

Pattern Approval Certificate of Measuring Instruments RU.C.31.058.A No.: 51345

Certificate of Conformity No.: РОСС RU.88.H00437, valid from Mar. 12, 2013 until Mar. 12, 2016, issued by the Federal Agency for Technical Regulation and Metrology